Imaging Workflow

- Open LAS X, set up channels (GFP, cFOS, DAPI, …)

- Under Acquisition mode select z

- Scroll down to the z-stack tab

- Select desired region using the rectangle

- Set the focus to the upper cells and click “Begin”

- Set the focus to the lower cells and click “End”

- Set z-step to 0.5

- After taking all the pictures, go to “Process” (next to Acquire) and select “Crop” (under “Edit”, which is under Process Tools), click “open project” and select one image

- Scroll through the images using the bar on the right and choose the starting image as the one that is the first one in focus and the last image as the last one in focus

- At the bottom in the middle, there is “z-stack-selection”, input your slice numbers, check that all channels are selected and click “Apply”

- Repeat for every image

- Then go to “Process” again, select “Lightning & Thunder process”

- Select all cropped images of one animal, right click, and select “send to…”, select “Batch processing”

- Choose the “Embryo” symbol, or hover over all symbols and select LVCC

- Mounting Medium: Fluoromount, Refractive Index 1.40000

- We used the standard parameters (if desired, change them under “advanced settings”)

- Click “Apply current settings to all jobs” and “Start Batch”

- Save the project

- Export thundered images as TIFF by selecting desired images, right click, “Export Image”, choose destination folder

- Check “Export LUT”, “Export Channels”, “Raw Image” and “Save as Multipage”

- Now you should have each image saved as three separate ones for each channel

- If that doesn’t work, go to another PC in the lab and export the pictures there